# Knowledge organiser · Plans, elevations and measures

GCSE Mathematics 1MA1 · L037 · 60 minutes

# Core knowledge

Plan = view from directly above.

Elevation = straight-on orthographic view from a named direction.

A 2D view records maximum visible extent in that direction; hidden depth may be lost.

Scale 1:100 means 1 unit on the drawing represents 100 of the same units in reality.

# Method checklist

Mark the viewing direction.

Use a square grid and align widths/heights.

For a plan, project occupied positions vertically from above.

For an elevation, remove perspective: use straight horizontal/vertical edges.

Check whether the information determines a unique solid.

For scale, convert to consistent units before area calculations.

# Common errors to avoid

Using perspective in an elevation; swapping front and side; assuming a plan shows height; applying the scale factor to area instead of converting lengths first.